PRODUCTION OF BUTYL ACETATE BY

REACTIVE DISTILLATION

THESIS

SUBMITTED TO THE PANJAB UNIVERSITY, CHANDIGARH

IN THE PARTIAL FULFILLMENT FOR THE DEGREE OF

MASTER OF SCIENCE

(INDUSTRIAL CHEMISTRY)

2010-2012

By

SHINU SINGLA

DEPARTMENT OF CHEMICAL ENGINEERING AND TECHNOLOGY,

PANJAB UNIVERSITY, CHANDIGARH – 160014

INDIA

University Institute of Chemical Engineering and Technology,

PANJAB UNIVERSITY,

Chandigargh-160014

Date: ______________

DECLARATION

I hereby declare that the work embodied in dissertation entitled “Production of butyl acetate by reactive distillation” is original piece of work and was conducted in University Institute of Chemical Engineering and Technology, Panjab University, Chandigarh. The matter presented in this thesis has not been submitted in part or full, to this or any other University/Institute for any degree or diploma.

Shinu Singla

University regd. No.: 1800700850

CERTIFICATE

This is to certify that the work being presented in this thesis entitled “Production of butyl acetate by reactive distillation”, submitted by Ms Shinu to University Institute of Chemical Engineering and Technology, Panjab University, Chandigarh in partial fulfillment of the requirements for the award of the degree of Master of Science in Industrial Chemistry is an authentic record of her own work carried out under my supervision. This report has not been submitted for the award of any other degree or certificate in this or any other University or Institute.

Date:

Dr. Ritu Gupta

Associate Prof., UICET

Panjab University

Chandigarh.

ACKNOWLEDGMENT

Before I present my thesis, I would like to add a few heartfelt words for the people who helped me in completing my thesis work. I am truly indebted and thankful to my guide Dr. Ritu Gupta, Department of Chemical Engineering and Technology, Panjab University, Chandigarh. I would like to express my special thanks of gratitude to my guide Dr. Ritu Gupta for her inspiration, interest, guidance and suggestions throughout the duration of the thesis.

I would like to thank Dr. R.K. Wanchoo, for his guidance throughout my work, as well as our chairperson Dr. Chhabra who gave me the excellent opportunity to do this wonderful work by providing me with necessary facilities.

I am deeply thankful to the laboratory and technical staff, especially Mr.Tulsi Ram (computer laboratory) for his help and assistance.

I am thankful to the entire staff members of the department of chemical engineering and technology, Panjab University, Chandigarh, and our librarian for their wonderful co-operation in completing my thesis work.

Moreover, I would like to thank my parents and friends who helped me a lot in finishing my thesis within the limited time.

I am thankful to all who helped me.

Shinu Singla

ABSTRACT

Butyl acetate is a widely known ester used as a solvent in the production of lacquers, more commonly as a synthetic apple flavoring used in foods such as ice cream, candy, jams and baked goods. The present study shows the esterification of acetic acid with n-butanol in the presence of heterogeneous catalyst Amberlyst-15 using batch distillation and batch reactive distillation. The kinetic behavior of the heterogeneous catalyst of acetic acid with n-butanol over an acidic cation exchange resin, Amberlyst-15 has also been investigated. The experiments were conducted in a batch distillation and batch reactive distillation at a temperature of 371.8K for different catalyst loading (3% - 17%) where the molar feed ratio of acetic acid and n-butanol was 1:1 (v/v). The conversion of acetic acid was found to increase with increasing catalyst loading up to 17%. The percentage enhancement in conversion was also observed.

LIST OF TABLES

| Table No. | Title | Page no. |
| --- | --- | --- |
| 3.1 | Physical and chemical properties of acetic acid | 9 |
| 3.2 | Physical and chemical properties of n-Butanol | 10 |
| 3.3 | Physical and chemical properties of amberlyst-15 | 11 |

LIST OF FIGURES

| Figure no. | Name of figures | Page no. |
| --- | --- | --- |
| 5.1 | Calibration curve | 21 |
| 5.2 | Plot of percentage conversion versus catalyst loading for batch reactive distillation | 22 |
| 5.3 | Plot of percentage conversion versus catalyst loading for batch distillation | 23 |
| 5.4 | Plot showing comparison between percentage conversion versus catalyst loading for batch reactive distillation and batch distillation | 24 |
| 5.5 | Plot for catalyst loading 3% at temperature 371.8K and molar ratio 1:1 | 25 |
| 5.6 | Plot for catalyst loading 5% at temperature 371.8K and molar ratio 1:1 | 25 |
| 5.7 | Plot for catalyst loading 7% at temperature 371.8K and molar ratio 1:1 | 26 |
| 5.8 | Plot for catalyst loading 9% at temperature 371.8K and molar ratio 1:1 | 26 |
| 5.9 | Plot for catalyst loading 11% at temperature 371.8K and molar ratio 1:1 | 27 |
| 5.10 | Plot for catalyst loading 13% at temperature 371.8K and molar ratio 1:1 | 27 |
| 5.11 | Plot for catalyst loading 15% at temperature 371.8K and molar ratio 1:1 | 28 |
| 5.12 | Plot for catalyst loading 17% at temperature 371.8K and molar ratio 1:1 | 28 |

NOTATIONS

ρ              Density of acetic acid

M.W        Molecular weight

M             Molar ratio

CAO          Initial concentration of acetic acid

CBO          Initial concentration of n-Butanol

T              Temperature

XA           Degree of fraction

W             Weight of catalyst

LIST OF CONTENTS                                                                            Page no.

CHAPTER – 1           INTRODUCTION……………………………………………….1- 3

  - Esters and their industrial applications…………….........................................2

    - Esterification catalysts and processes………………………….....3

1.1.1.1 Homogenous acid catalysts………………………………..3

1.1.1.2 Heterogeneous acid catalysts……………………………..3

CHAPTER – 2            LITERATURE REVIEW……………………………………….5-7

CHAPTER – 3            MATERIALS AND METHODS………………………………..8-16

3.1 Apparatus……………………………………………………………………...8

3.2 Chemicals……………………………………………………………………...8

3.3 Acetic Acid…………………………………………………………………….8

3.4 n-Butanol………………………………………………………………………9

3.5 Catalyst………………………………………………………………………...11

3.5.1 Catalyst modification………………………………………………..11

3.5.2 Regeneration of the catalyst…………………………………………11

3.6 Preparation of solutions………………………………………………………..12

3.6.1 Feed solution………………………………………………………..12

3.6.1.1 For (v/v) acetic acid solution……………………………..12

3.6.1.2 For (v/v) n-Butanol solution………………………………12

3.6.1.3 Std. NaOH solution………………………………………..13

3.6.1.4 Std. oxalic acid solution……………………………………13

3.7 Methodology…………………………………………………………………13

3.8 Experimental set up………………………………………………………….14

3.8.1 Batch distillation……………………………………………..14

3.8.2 Batch reactive distillation……………………………………14

3.9 Procedure……………………………………………………………………15

CHAPTER – 4            SAMPLE CALCULATIONS…………………………………….17-19

CHAPTER – 5             RESULTS AND DISCUSSION…………………………………..20-28

5.1 Calibration curve…………………………………………………………….20

5.2 Results………………………………………………………………………..21

5.3 Discussion……………………………………………………………………22

5.4 Kinetic model………………………………………………………………...24

5.5 Conclusion…………………………………………………………………...28

APPENDIX……………………………………………………………………………………29-45

REFRENCES…………………………………………………………………………………46